Obstacle

from controller import Robot

robot = Robot()

timestep = int(robot.getBasicTimeStep())

# Get motors

left_motor = robot.getDevice('left wheel motor')

right_motor = robot.getDevice('right wheel motor')

# Set motors to velocity mode

left_motor.setPosition(float('inf'))

right_motor.setPosition(float('inf'))

# Initial speed

left_motor.setVelocity(0.0)

right_motor.setVelocity(0.0)

# Distance sensors

sensor_names = [

'ps0', 'ps1', 'ps2', 'ps3',

'ps4', 'ps5', 'ps6', 'ps7'

]

sensors = []

for name in sensor_names:

sensor = robot.getDevice(name)

sensor.enable(timestep)

sensors.append(sensor)

MAX_SPEED = 6.28

TURN_SPEED = 3.0

THRESHOLD = 80

while robot.step(timestep) != -1:

values = [s.getValue() for s in sensors]

# --- KEY FIX BELOW ---

# ps7 is Front-Left, ps0 is Front-Right

# ps6 and ps1 are angled slightly, good for corners

# Check the right side of the front

right_obstacle = values[0] > THRESHOLD or values[1] > THRESHOLD

# Check the left side of the front

left_obstacle = values[7] > THRESHOLD or values[6] > THRESHOLD

if left_obstacle:

# Obstacle on the left? Turn Right.

left_motor.setVelocity(TURN_SPEED)

right_motor.setVelocity(-TURN_SPEED)

elif right_obstacle:

# Obstacle on the right? Turn Left.

left_motor.setVelocity(-TURN_SPEED)

right_motor.setVelocity(TURN_SPEED)

else:

# No obstacle? Go Forward

left_motor.setVelocity(MAX_SPEED)

right_motor.setVelocity(MAX_SPEED)

Wall

from controller import Robot

robot = Robot()

timestep = int(robot.getBasicTimeStep())

left_motor = robot.getDevice('left wheel motor')

right_motor = robot.getDevice('right wheel motor')

left_motor.setPosition(float('inf'))

right_motor.setPosition(float('inf'))

# --- TUNING PARAMETERS ---

MAX_SPEED = 10.28

CRUISE_SPEED = 4.0

TARGET_WALL_DIST = 150.0  # Distance to keep from wall

KP = 0.02                 # LOWERED THIS to stop the zig-zag

DEADZONE = 5.0            # If error is less than 5, do nothing

sensor_names = ['ps0', 'ps1', 'ps2', 'ps3', 'ps4', 'ps5', 'ps6', 'ps7']

sensors = []

for name in sensor_names:

sensor = robot.getDevice(name)

sensor.enable(timestep)

sensors.append(sensor)

while robot.step(timestep) != -1:

values = [s.getValue() for s in sensors]

# Front sensors (to detect walls in front)

front_wall = values[0] > 80 or values[7] > 80

# Left sensor (to follow the wall)

left_sensor_value = values[5]

if front_wall:

# Sharp turn right if wall is in front

left_motor.setVelocity(2.0)

right_motor.setVelocity(-2.0)

else:

# WALL FOLLOWING LOGIC

error = TARGET_WALL_DIST - left_sensor_value

# 1. Apply Deadzone: if the error is tiny, ignore it to stay straight

if abs(error) < DEADZONE:

correction = 0

else:

# 2. Apply a small correction

correction = error * KP

# Calculate speeds

v_left = CRUISE_SPEED - correction

v_right = CRUISE_SPEED + correction

# Clamp speeds to limits

v_left = max(min(v_left, MAX_SPEED), -MAX_SPEED)

v_right = max(min(v_right, MAX_SPEED), -MAX_SPEED)

left_motor.setVelocity(v_left)

right_motor.setVelocity(v_right)